High resolution License Plate Recognition Camera for Highway

Description：

There are  two model series about the LRP camera: white LEDs with short distance and slow speed for parking lot application, and IR LEDs with long distance and fast speed for traffic.     Features：

1.The LPR cameras adopts high resolution CCD, the new ISP and high speed MCU design, which are very easy to use due to the wired button or RS485 different models and LED brightness adjusting. The optional model using with ground sensor can capture the license plate when the car is in and monitor the normal environment when the car pass away.

2.The built in fan and heater, double layer waterproof housing is weather-sealed and field proven for successful application in extreme environments.

3.  Six kinds of mode selection with easy-operation

Specification:

| Product name | High Resolution LPR Camera series |
| --- | --- |
| Video system | PAL/NTSC |
| Sensor | 1/3” SONY 960H Ex-view HAD II CCD |
| Pixel | PAL:976(H)x 582(V)/NTSC:976(H)x 494(V) |
| Resolution | Color:650TVL B/W: 700TVL |
| Video output | 1.0Vp-p/75Ω Video composite video signal |
| Capture mode | Modes1-6 selectable, display in the monitor |
| Addition function (Ground sensor model) | LPR function when the car is in, normally monitor when the car pass away, ground sensor model |
| LED | 4pcs IR Led |
| Lens | 5-50mm DC lens |
| Day/night | IR LED , B/W image and ICR auto switching for IR model |
| Power Supply | DC12V (±10%), 1A at DC12V(Max) |
| Work environment | -30degree~70degree,20%~90% RH |
| Temperature control | Heater works at –5degree(optional), fan works at 45degree |
| Dimension | 290x147x101(mm) |